Appendix 3

Infection Prevention Audit:  Practice Checklist for Minor Surgery

| Questions | Compliance (if applicable) | Comments | Rationale |
| --- | --- | --- | --- |
| There is a room designated for minor surgery |  |  | Expert Guidance |
| The minor surgery environment is uncluttered with adequate storage space. |  |  | Best Practice |
| The minor surgery room floor covering is intact, washable, nonslip with coved edges and able to withstand chemical disinfection. |  |  | Best Practice |
| The minor surgery room walls have smooth, washable surfaces (no ceramic tiles) |  |  | Best Practice |
| The ceilings in the minor surgery room have smooth washable surfaces. |  |  | Best Practice |
| The window in the minor surgery room ensures privacy with opaque glass (no curtains). |  |  | Best Practice |
| The ceiling light in the minor surgery room is covered. |  |  | Best Practice |
| There is adequate ventilation in the minor surgery room by natural or mechanical means |  |  | Expert Guidance |
| Mechanical extract/ventilation in the minor surgery undergoes routine cleaning. |  |  | Best Practice |
| The heat source in the minor surgery room e.g. the radiator (and pipework) is contained in a smooth-surfaced washable box |  |  | Best Practice |
| The work surfaces in the minor surgery room are smooth, impervious and able to withstand chemical disinfectants |  |  | Best Practice |
| There is no open shelving in the minor surgery room. |  |  | Best Practice |
| The treatment couch is intact with a washable, impervious surface. |  |  | Expert Guidance |
| The couch is protected with disposable paper towel changed between service users. |  |  | Best Practice |
| There is a cleaning schedule for privacy curtains |  |  | Best Practice |
| Clinical hand wash basins conform to current recommended guidance (HTM 64): - Elbow, foot or infra-red operated mixer taps - No swan-neck fittings on taps - Thermostatically controlled hot water - Free from sink plugs and overflow - Water faucet extends over the drainage outlet |  |  | Expert Guidance |
| The minor surgery room has a wall mounted dispenser containing disposable paper hand towels and a liquid soap dispenser. |  |  | Best Practice |
| The minor surgery room's clinical hand wash basin is free from reusable nail brushes. |  |  | Expert Guidance |
| Skin antiseptics (e.g. Chlorhexidine, Betadine) are available for aseptic hand washing. Antiseptics are wall mounted and dispensed by a plunger or infra-red |  |  | Best Practice |
| Antiseptic aqueous based skin preparations available i.e. Chlorhexidine, iodine |  |  | Best Practice |
| Single use, powder-free sterile surgeons gloves available if required. |  |  | Legislation PPE Regulations |
| The following are available when splashing of body fluids is anticipated: Plastic goggles OR, Fluid repellent face mask/goggles/visor. |  |  | Best Practice |
| The use of disposable equipment is recommended or Sterile instrument packs are available and supplied by SSD |  |  | Best Practice |
| The minor surgery room has disposable sterile drapes available. |  |  | Best Practice |
| The minor surgery room has a designated stainless steel procedure trolley |  |  | Expert Guidance |
| Dressing trolleys in the minor surgery room are cleaned with detergent and water before each session and with detergent and water or 70% alcohol between cases. |  |  | Expert Guidance |
| The minor surgery room has access to an adjacent dirty utility area |  |  | Best Practice |
| The minor surgery room has a designated decontamination sink in the dirty utility |  |  | Best Practice |
| Waste bags are not attached to cupboard/trolley etc. |  |  | Best Practice |
| The minor surgery room has a foot operated, lidded clinical waste bin with yellow/orange bag. |  |  | Best Practice |
| The minor surgery room has a securely positioned sharps container which conforms to BS7320/UN3291 |  |  | Best Practice |
| Clinical areas in the minor surgery room are cleaned at the beginning and end of each minor surgery clinic with an appropriate detergent |  |  | Expert Guidance |
| The minor surgery room has a domestic cleaning schedule that documents twice daily cleaning. |  |  | Best Practice |
| Following minor surgery, service users should return within two weeks for a follow up appointment following a minor surgical procedure if clinically appropriate. |  |  | Best Practice |
| The minor surgery clinic has a surveillance system for post-operative wound infection |  |  | Best Practice |

Signature:  __________________________________________________________

Date: __________________________________________